How to program for the video mode

- Initialization

INT 10h  With AH=00h, AL= Mode Number

A full list of modes can be found at：

http://www.columbia.edu/~em36/wpdos/videomodes.txt

With AL=13h, you get: 	 320x200, 256 Colors

- Modify the Screen Buffer

The screen buffer address is listed in the above file.

For AL=13h, the buffer starts from A000h:0000h

Each pixel takes one of the 256 colors, which can be coded into one byte.

The screen is 320*200*1 = 64000 bytes – just fits into one segment.

Pixels are stored from left to right, from top to bottom.

- First Row: 	From A000:0000 to A000:013F

- Second Row: 	From A000:0140 to A000:027F

Thus, the color of the pixel at (x,y) is stored at:

A000: [(320 * Y) + X]

The screen starts from (0,0), NOT (1,1)!!!

For 8255 PPI， visit:

http://www.advancedmsinc.com/iocards/8255.htm

For Programmable Interrupt Timer:

Default Mode is Mode 3.

- Mode 3 is typically used for Baud rate generation.

- OUT will initially be high.

- When half the initial count has expired, OUT goes low for the remainder of the count.

- Mode 3 is periodic; the sequence above is repeated indefinitely.

- An initial count of N results in a square wave with a period of N CLK cycles.