## What are we going to do with the UART?

## Transmit or receive data – Continuous data stream

## Control the protocol parameters – Initialization

Control the baud rate - Initialization

### [To Write to a Port]

### MOV AL, data

### MOV DX, portnum

### OUT DX, AL

### [To Read From a Port]

### MOV DX, portnum

### IN AL, DX

## Handshaking – Checking & Asserting

## Registers for Serial Port (IO Port, Not CPU Registers)

Eight I/O bytes are used for each UART to access its registers.

The switch bit DLAB can be found in the line control register LCR as bit 7 at I/O address base + 3.

| Register to port map |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | DLAB = 0 |  | DLAB = 1 |  |
| I/O port | Read | Write | Read | Write |
| Base (3F8h) or (2F8h) | RBR☆ Receiver buffer | THR☆ Transmitter holding | DLL (divisor latch LSB) |  |
| base + 1 | IER interrupt enable | IER interrupt enable | DLM (divisor latch MSB) |  |
| base + 2 | IIR Interrupt identification | FCR FIFO control | IIR Interrupt identification | FCR FIFO control |
| base + 3 | LCR☆ line control |  |  |  |
| base + 4 | MCR☆ modem control |  |  |  |
| base + 5 | LSR☆ Line status | factory test | LSR Line status | factory test |
| base + 6 | MSR☆ Modem status | not used | MSR Modem status | not used |
| base + 7 | SCR scratch |  |  |  |

The communication between the processor and the UART is completely controlled by the 12 registers in the I/O address area.

### RBR : Receiver buffer register (RO) – Data Buffer

### THR : Transmitter holding register (WO) – Data Buffer

### MSR: Modem status register (RO) – Status - Handshaking

### MCR: Modem control register (R/W) – Control - Handshaking

### LSR: Line status register (RO) - Status – Error and Transmission

### LCR : Line control register (R/W) － Control - Initiallization

| LCR : line control register |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Bit | Value |  |  |  | Comment |
| 0,1 | Bit 1 |  | Bit 0 |  | Data word length |
|  | 0 |  | 0 |  | 5 bits |
|  | 0 |  | 1 |  | 6 bits |
|  | 1 |  | 0 |  | 7 bits |
|  | 1 |  | 1 |  | 8 bits |
| 2 | 0 |  |  |  | 1 stop bit |
|  | 1 |  |  |  | 1.5 stop bits (5 bits word) 2 stop bits (6,7 or 8 bits word) |
| 3,4,5 | Bit 5 | Bit 4 |  | Bit 3 |  |
|  | X | x |  | 0 | No parity |
|  | 0 | 0 |  | 1 | Odd parity |
|  | 0 | 1 |  | 1 | Even parity |
|  | 1 | 0 |  | 1 | High parity (stick) |
|  | 1 | 1 |  | 1 | Low parity (stick) |
| 6 | 0 |  |  |  | Break signal disabled |
|  | 1 |  |  |  | Break signal enabled |
| 7 | 0 |  |  |  | DLAB : RBR, THR and IER accessible |
|  | 1 |  |  |  | DLAB : DLL and DLM accessible |

| ### LSR : Line status register |  | ### MSR : Modem status register |  | ### MCR : Modem control register |  |
| --- | --- | --- | --- | --- | --- |
| Bit | Comment | Bit | Comment | Bit | Comment |
| 0 | Data available | 0 | delta Clear to send | 0 | Data terminal ready |
| 1 | Overrun error | 1 | delta Data set ready | 1 | Request to send |
| 2 | Parity error | 2 | trailing edge Ring indicator | 2 | Auxiliary output 1 |
| 3 | Framing error | 3 | delta Carrier detect | 3 | Auxiliary output 2 |
| 4 | Break signal received | 4 | Clear to send | 4 | Loopback mode |
| 5 | THR is empty | 5 | Data set ready | 5 | Autoflow control |
| 6 | THR empty, line idle | 6 | Ring indicator | 6 | Reserved |
| 7 | Erroneous data in FIFO | 7 | Carrier detect | 7 | Reserved |

Refer to Sect#22.3 of AoALP for Serial Port Example Programs

Baud & Bits Per Second (BPS)

- The two most corrupted terms in telecommunications.

- Baud is the number of signal level changes per second in a line, regardless of the information content of those signals.

- Bits per second is the rate of transfer of information bits.

- The ratio of BPS to baud depends on the information coding scheme that you are using.

- For example, each character in asynchronous RS-232 coding includes a start and stop bit that are not counted as information bits, so the BPS rate is actually less than the baud rate.

- Present-day modems, on the other hand, use a set of discrete amplitude and phase values to encode multiple bits with each signal change. This technique, along with compression schemes and other tricks, allows BPS rates of 14,400 and higher on lines that support relatively low signal change rates (about 2400 baud for phone lines).

### Devisor Setting: Divisor value = Input frequency / (baud rate * 16)

### Interrupt of 8086/8088

### The 8086 series of microprocessors has an Interrupt Vector Table situated at 0000:0000 which extends for 1024 bytes.

### The Interrupt Vector table holds the address of the Interrupt Service Routines (ISR), all four bytes in length.

### This gives us room for the 256 Interrupt Vectors.

| INT (Hex) | IRQ | Common Uses |
| --- | --- | --- |
| 00 - 01 | Exception Handlers | - |
| 02 | Non-Maskable IRQ | Non-Maskable IRQ (Parity Errors) |
| 03 - 07 | Exception Handlers | - |
| 08-0F | Hardware IRQ0~7 | System Timer, Keyboard, Serial Comms, Sound Card, FD Ctrl, Parallel Comms. |
| 10 - 6F | Software Interrupts | (DOS INT, e.g. 21H) |
| 70 - 77 | Hardware IRQ8~15 | Real Time Clock, Mouse, HD Drive, Co-Processor |
| 78 - FF | Software Interrupts | - |

### Go online for BIOS and DOS ISR list: http://ee2007.cjb.net/onlinepages.html

### Example Description

## Int 21/AH=01h: READ CHARACTER FROM STDIN, WITH ECHO

AH = 01h

Return:  AL = character read

Notes: ^C/^Break are checked, and INT 23 executed if read. …

### CODE:

### MOV AH, 01h

### INT 21H

### MOV DL, AL

### Parallel Port is more straight-forward

### Port Register value mapped directly to connector pins (0V or 5V)

| ### Bit | ### Data Port (378h) |  | ### Status Port (379h) |  | ### Control Port (37Ah) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ### Func. | ### Pin. | ### Func. | ### Pin | ### Func. | ### Pin |
| ### D0 | ### Data0 | ### 2 | ### Not Used | ### - | ### STROBE | ### 1 |
| ### D1 | ### Data1 | ### 3 | ### Not Used | ### - | ### Auto FD | ### 14 |
| ### D2 | ### Data2 | ### 4 | ### Not Used | ### - | ### /INIT | ### 16 |
| ### D3 | ### Data3 | ### 5 | ### /ERROR | ### 15 | ### /SLCT_IN | ### 17 |
| ### D4 | ### Data4 | ### 6 | ### /SLCT | ### 17 | ### IRQ7 | ### - |
| ### D5 | ### Data5 | ### 7 | ### PE | ### 12 | ### Not Used | ### - |
| ### D6 | ### Data6 | ### 8 | ### /ACK | ### 10 | ### Not Used | ### - |
| ### D7 | ### Data7 | ### 9 | ### /BUSY | ### 11 | ### Not Used | ### - |

ComRead	proc

Push	dx

Call	GetLCRCom

Push	ax	;Save DLAB.

And	al, 7fh	;Select normal ports.

Call	SetLCRCom	;Write LCR to turn off DLAB

WaitForChar:

Call	GetLSRCom	;Get LSR Register

Test	al, 00000001b   ;Data Available?

Jz	WaitForChar	;Loop until data available.

Mov	dx, comport	;Read from the input port.

In	al, dx

Mov	dl, al	;Save character

Pop	ax	;Restore DLAB

Call	SetLCRCom	;Write it back to LCR.

Mov	al, dl	;Restore output character.

Pop	dx

ret

ComRead	endp

ComWrite	proc

Push	dx

Push	ax

Mov	dl, al	;Save character to output

Call	GetLCRCom	;Switch to output register.

Push	ax	;Save divisor latch access bit.

And	al, 7fh	;Select normal i/o ports

Call	SetLCRCom	; rather than divisor reg.

WaitForXmtr:

Call	GetLSRCom	;Read LSR for xmit empty bit.

Test	al, 00100000b   ;Xmtr buffer empty?

Jz	WaitForXmtr	;Loop until empty.

Mov	al, dl	;Get output character.

Mov	dx, ComPort	;Store it in the ouput port to

Out	dx, al	; get it on its way.

Pop	ax	;Restore divisor access bit.

Call	SetLCRCom

Pop	ax

Pop	dx

Ret

ComWrite	endp

ComGetLCR	proc		;Return LCR value in AL.

Push	dx

Mov	dx, comLCR	;Point at LCR register.

In	al, dx	;Read and return LCR value.

Pop	dx

Ret

ComGetLCR	endp

ComSetLCR	proc		;Write a new value to the LCR.

Push	dx

Mov	dx, comLCR	;Point at LCR register.

Out	dx, al	;Write value in AL to the LCR.

Pop	dx

Ret

ComSetLCR	endp