Programmable Interval Timer

- What is it?

- Yet another chip connected to the processor through the BUSes.

- Accessed through I/O ports. (IN/OUT instructions)

- A chip to help the processor

- Timer / Counter

- How do you deal with time (delay) in your program?

    - Delay Loops

    - System Functions

    - …? PIT?

      - Generation of accurate time delays under software control

      - Minimal software overhead

      - Variable length delays can easily be accommodated

- PIT is simply three identical counters. But what is a counter?

      - What is EE2006? Remember?

      - Count clock signals ---- Input CLK

      - Numerical States ---- Q7~Q0

      - Output Signal ----- To indicate a special state

      - The counters in PIT are under software control --- The Modes

      - Configurable Output Signal

      - Initial Count Configurable

      - Flexible Monitoring

- How to program the PIT?

- Four Ports ( One for each counter, And One for Control)

- Write to configure the counters

- Read to monitor the counters

- How to Configure the PIT?  ( Write )

- What to configure?

  - Initial Counts	-- Each counter has a dedicated port.

  - Control Word	-- Configuration for all the counters

- Sequence of Operation

  - For each Counter, the Control Word must be written before the initial count is written.

  - The initial count must follow the count format specified in the Control Word (least significant byte only, most significant byte only, or least significant byte and then most significant byte).

- Definition of the Control Word

- Each Bit Described

SC - Select Counter

| SC1 | SC0 |  |
| --- | --- | --- |
| 0 | 0 | Select Counter 0 |
| 0 | 1 | Select Counter 1 |
| 1 | 0 | Select Counter 2 |
| 1 | 1 | Read-Back Command |

RW - Read/Write

| RW1 | RW0 |  |
| --- | --- | --- |
| 0 | 0 | Counter Latch Command (See Read Operations) |
| 0 | 1 | Read/Write least significant byte only. |
| 1 | 0 | Read/Write most significant byte only. |
| 1 | 1 | Read/Write least significant byte first, then most significant byte. |

M – Mode

| M2 | M1 | M0 |  |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Mode 0 |
| 0 | 0 | 1 | Mode 1 |
| X | 1 | 0 | Mode 2 |
| X | 1 | 1 | Mode 3 |
| 1 | 0 | 0 | Mode 4 |
| 1 | 0 | 1 | Mode5 |

BCD - Binary Coded Decimal

| 0 | Binary Counter 16-bit |
| --- | --- |
| 1 | Binary Coded Decimal (BCD) Counter (4 Decades) |

- How to monitor the counters? ( Read )

- What to monitor?

  - The current count

  - The current configuration

- A big problem: the Counters are always counting!

- Latch it and Read it

  - First Write to the Control Word to latch certain counter

  - Then read it through the port

  - If you latch more than once, all ONLY the FIRST is latched.

- How many ways to read?

- Three ways:

  - Direct Read

  - Using “Counter Latch Command”

  - Using “Read Back Command”

- Direct Read is not safe.

- Counter Latch Command

  - Write the following word to Control Word Port:

  - Read each counter port to get the latched count.

- Read Back Command

  - Write the following word to Control Word Port:

  - /COUNT and /STATUS indicates what to latch

  - CNT2~CNT0 indicates which counter(s) to latch

  - Equivalent to several counter latch commands, one for each counter latched.

  - You can latch Status and Count together.

    - In this case, you have to read the counter port twice

    - The first byte you get will be the STATUS

    - The second byte you get will be the COUNT.

  - The STATUS is defined as below:

- What are the modes?  (Read the Datasheet)

- Altogether SIX modes

- Defines how the OUTPUT signal is generated.

- GATE is an input signal usually used as gate or trigger.

- For example: Mode 0 --- Interrupt on Terminal Count

  - First load Control Word and Initial Count

  - CLK input will be counted and the counter decreased.

  - When decreased to zero, OUTPUT signal is asserted.

  - GATE signal will gate the CLK. (Like an AND Gate)

- Designated functions of each counter in PC.

- Counter 0: To generate INT 8. A time interrupt that appears 18.2 times per second. (Mode 3)

- Counter 1: To generate RAM refreshing clock. (Mode 2)

- Counter 2: To generate tones for the speaker. (Mode 3)

- Sample Code

cli

mov  al,00110110b  ; bit 7,6 = (00) timer counter 0

; bit 5,4 = (11) write LSB then MSB

; bit 3-1 = (011) generate square wave

; bit 0 = (0) binary counter

out  43h,al        ; prep PIT, counter 0, square wave&init count

jmp  $+2

mov  cx,countdown  ; default is 0x0000 (65536) (18.2 per sec)

; interrupts when counter decrements to 0

mov  al,cl         ; send LSB of timer count

out  40h,al

jmp  $+2

mov  al,ch         ; send MSB of timer count

out  40h,al

jmp  $+2

sti